DH542

Ultra low noise , high stability and smoothness digital stepper drive

Introduction

Overview

DH542 is the new generation high performance fully digital stepper drive which base on 32bit DSP with a advanced algorithm .the power supply range is DC20~50V. it can drive frame 42---57mm 2 phase stepper motor .

Ultra low noise and high smoothness at middle/low speed ,and high torque at high speed, which can be applied to different speed control applications. adopt smooth and accurate sinusoidal current vector control technology ,which make motor low heat .

Features

●new generation 32bit DSP

●good smoothness ,low noise ,low heat

●build-in micro step setting

●build-in current setting

●the standstill current can be set to 50%

●opto-isolated differential pulse input

● the response frequency is up to 300KHz

●over voltage , under voltage, over current protection

Applications

It can be used in various kinds of machines ,such as dispenser machine. floating machine

3D printer, labelling machine, medical machine, laser cut machine, and so on ,its unique features make it an ideal solution for the applications which require low noise ,low vibration, low heat and high precision

Mounting dimension（unit：mm）

Function

Current Setting

| Peak current(A) | RMS current (A) | SW1 | SW2 | SW3 |
| --- | --- | --- | --- | --- |
| 1.0 | 0.7 | OFF | OFF | OFF |
| 1.4 | 1.0 | ON | OFF | OFF |
| 1.9 | 1.4 | OFF | ON | OFF |
| 2.4 | 1.7 | ON | ON | OFF |
| 2.8 | 2.0 | OFF | OFF | ON |
| 3.2 | 2.3 | ON | OFF | ON |
| 3.8 | 2.7 | OFF | ON | ON |
| 4.2 | 3.0 | ON | ON | ON |

Standstill Current Setting

SW4：OFF=Half Current

ON=Full Current

Microstep Setting

| Pul/rev | SW5 | SW6 | SW7 | SW8 |
| --- | --- | --- | --- | --- |
| 200 | ON | ON | ON | ON |
| 400 | OFF | ON | ON | ON |
| 800 | ON | OFF | ON | ON |
| 1600 | OFF | OFF | ON | ON |
| 3200 | ON | ON | OFF | ON |
| 6400 | OFF | ON | OFF | ON |
| 12800 | ON | OFF | OFF | ON |
| 25600 | OFF | OFF | OFF | ON |
| 1000 | ON | ON | ON | OFF |
| 2000 | OFF | ON | ON | OFF |
| 4000 | ON | OFF | ON | OFF |
| 5000 | OFF | OFF | ON | OFF |
| 8000 | ON | ON | OFF | OFF |
| 10000 | OFF | ON | OFF | OFF |
| 20000 | ON | OFF | OFF | OFF |
| 25000 | OFF | OFF | OFF | OFF |

Interface Description

| Name | Function | Description |
| --- | --- | --- |
| PWR | Power light | When power on ,the green light is on |
| ALARM | Over current/under voltage/over voltage displaying light | If the drive Over current/under voltage/over voltage, the red light is on |
| PU+ | Pulse input positive | Pulse signal: In single pulse (pulse/direction) mode, this input represents pulse signal, each falling edge active; 4-5V for High -level, 0-0.5V for low-level Series connect resistors for current-limiting when +12V or +24V used.(should connect 1k resistor for +12V,2k for +24V) |
| PU- | Pulse input negative | Pulse signal: In single pulse (pulse/direction) mode, this input represents pulse signal, each falling edge active; 4-5V for High -level, 0-0.5V for low-level |
| DR+ | Direction input positive | DIR signal: In single-pulse mode, this signal has low/high voltage levels, representing two directions of motor rotation; Series connect resistors for current-limiting when +12V or +24V used.(should connect 1k resistor for +12V,2k for +24V) |
| DR- | Direction input negative | DIR signal: In single-pulse mode, this signal has low/high voltage levels, representing two directions of motor rotation; |
| MF+ | Enable input positive | Enable signal: This signal is used for enabling/disabling driver. High level for enabling the driver and low level for disabling the driver. Usually left UNCONNECTED (ENABLED). Series connect resistors for current-limiting when +12V or +24V used.(should connect 1k resistor for +12V,2k for +24V) |
| MF- | Enable input negative | Enable signal: This signal is used for enabling/disabling driver. High level for enabling the driver and low level for disabling the driver. Usually left UNCONNECTED (ENABLED). |
| V- | Power input COM | DC20-50V |
| V+ | Power input VCC |  |
| A+,A- | Motor A phase | Connect to Motor A phase |
| B+,B- | Motor B phase | Connect to Motor B phase |

！Note

A: Connect power supply rightly ,can’t exceed 50VDC

B: If the control signal (PU, DR,MF ) is over 5V ,please connect resistors (at the PU+, DR+, MF+ terminals) for current-limiting.

C:if the alarm light is on ,please check it after power off

a :the power supply range is 20V—50VDC or not ? b:is there any short on motor wire or others ?