HS806  Hybrid servo (closed loop stepper drive )

Introduction

Overview

HS806  Hybrid servo (closed loop stepper drive ) is based on the latest DSP technology and  advanced control algorithm of combing brushless servo and stepper systems. It combines favored features of both open loop stepper and brushless servo drives, solve the fatal problem “loss of steps” of open loop step systems ,improve the stepper motor high speed performance, and  motor & drive heating and noise are also significantly improved.

Compared with brushless servo systems, they have much higher starting and low-speed torque, high standstill stiffness, no hunting, no overshooting, almost zero settling time, and no tuning for almost for all applications.

Feature

●all digital vector control

●full close loop control ,no lost step

●high speed ,the max speed up to 4500RPM

●fast start/stop, no ACC/DEC to 3000rpm

●good performance at smoothness ,low noise and vibration

●about 100% power output, low heat

●the response frequency is 300KHZ

●overcurrent , undervoltage, overvoltage, position error protection

Applications

The main applications are cnc router, plasma/flame cutting, robot ,laser , advertising equipment, Packaging Equipment, etc.

Mounting（unit：mm）

Function description

DIP settings

SW1：current loop setting，OFF：low value；ON：high value

SW2：position loop，OFF：low value；ON：high value

SW3：the max peak current setting:  OFF：the max current 5.0A；

ON：the max current 6.5A

SW4：motor direction，OFF：CW；ON：CCW

SW5~SW8：Micro step setting，

| Pul/rev | SW5 | SW6 | SW7 | SW8 |
| --- | --- | --- | --- | --- |
| 400 | ON | ON | ON | ON |
| 800 | OFF | ON | ON | ON |
| 1600 | ON | OFF | ON | ON |
| 3200 | OFF | OFF | ON | ON |
| 6400 | ON | ON | OFF | ON |
| 12800 | OFF | ON | OFF | ON |
| 25600 | ON | OFF | OFF | ON |
| 51200 | OFF | OFF | OFF | ON |
| 1000 | ON | ON | ON | OFF |
| 2000 | OFF | ON | ON | OFF |
| 4000 | ON | OFF | ON | OFF |
| 5000 | OFF | OFF | ON | OFF |
| 8000 | ON | ON | OFF | OFF |
| 10000 | OFF | ON | OFF | OFF |
| 20000 | ON | OFF | OFF | OFF |
| 40000 | OFF | OFF | OFF | OFF |

Interface

| Name | Function | Description |
| --- | --- | --- |
| POWER | Power light | When power on ,the light is on |
| ALARM | Overcurrent/undervoltage/overvoltage light | When the drive show alarm, the light is on |
| PLS+ | Pulse + | Pulse signal, the range is +5V-+24V,if more than+5V,it need connect resistor |
| PLS- | Pulse- | Low level is 0-0.5V,high level is 4-5V,pulse width >2us。 |
| DIR+ | Direction+ | Direction signal, the range is +5V-+24V,if more than+5V,it need connect resistor |
| DIR- | Direction- | Low level is 0-0.5V,high level is 4-5V,pulse width >2us。 |
| ENA+ | Motor enable | the range is +5V-+24V,if more than+5V,it need connect resistor |
| ENA- | Motor Enable- | Low level is effective ,if it’s effective ,the dirve stop working, the motor disable |
| PEND+ | Position end+ | Motor reach the specified position ,output a signal. PEND+ connect to Vcc, PEND- connect to the controller input. The max current is 50MA |
| PEND- | Position end- |  |
| ALM+ | Alarm+ | When the drive show alarm, the alarm output is on .ALM+ connect to Vcc, ALM– connect controller input。The Max current is 50MA |
| ALM- | Alarm- |  |
| PB+/PB- | Encoder B phase | Encoder B phase input |
| PA+/PA- | Encoder A phase | Encoder A phase input |
| VCC/GND | Encoder Power supply | Encoder Power supply(5V) |
| AC1 | AC power input | AC24-80V |
| AC2 | AC power input |  |
| A+,A- | Motor A phase | Motor A phase，the phase sequence should be correct |
| B+,B- | Motor B phase | Motor B phase, the phase sequence should be correct |

！Notice

A、the input power supply can not over AC80V。

B、The command signal is 5V ,if it is more than 5V ,please connect resistor

C、when the Alarm light is on ,please power off and check

a. the power range is 24VAC---80VAC ；

b. the motor wiring and encoder wiring ,check is there any short